Chips and Technologies, Inc.

Graphics Software Group

October 6, 1994

Release Notes

65540/545 Demo/Utilities Release 1.1

Chips and Technologies, Inc. is providing our customers with the demonstration and utilities programs:

54XDEBUG (Ver 1.0) is TSR debug program which facilitates reading and writing the 65540/545's registers and BIOS data and configuring the 65540/545's features.

54XSET (Ver 1.0) is a command line program that enables end-users to turn on/off various 65540/545 graphics controller features.

MODETEST (Ver 3.5) is a diagnostic to set and display information for each video mode.

SETCLK (Ver 2.0) is a utility to re-program the 65540/545's MCLK and PCLK (dot clock) frequencies generated by the 65540/545 clock synthesizer (e.g.,  any value:  12, 13..., 100 MHz etc.)

SETVGA (Ver 1.0.8) is a utility to set EGA, CGA, MDA or Hercules emulation mode.

DEBUGVGA (Ver 2.7) is a utility that reads, writes, dumps and loads the 65540/545's registers, sets VGA or SuperVGA BIOS modes and computes CRT timings for the current mode.

54XDEMO (Ver 1.0) is a menu-driven, user-interactive demonstration that explains the various features of the 65540/545 VGA flat panel/CRT controller.

Installation

To copy all the 65540/545 Demo/Utilities files from diskettes in the floppy drive to a single sub-directory on the hard drive, type the following command at the command prompt:

Usage: 	INSTALL [destination pathname]

where [destination pathname] is the pathname of the directory (on the current drive) into which the 	65540/545 Demo/Utilities will be installed.

Example: 	INSTALL C:\54XDEMO

The designated sub-directory should be added to the PATH statement in the AUTOEXEC.BAT file to allow these programs to be used easily from other directories.

54XDEBUG TSR Debug Utility Program

To install 54XDEBUG as a TSR program, type the following command at the command prompt:

Usage: 	54XDEBUG

After it's been installed as a TSR, to invoke the 54XDEBUG program during any application program (except Windows, which has problems with TSRs), hold down both the <Left SHIFT> key and the <Right SHIFT> key at the same time.  The 54XDEBUG's Main Menu should be displayed in the upper left-hand corner of the screen as a window in the application program.  The 54XDEBUG's Main Menu consists of four sub-menus:

Sub-Menu	Function

Registers Menu	Read/Write All 65540/545 Register Values

Bios Data Area	Read All 65540/545 Video BIOS Data

Features Menu	Enable/Disable Various 65540/545 Features

Exit 	Exit the TSR program

To read or write any of the 65540/545's register values:

1)	Highlight (by moving the inverse video box with the <UP ARROW> or <DOWN ARROW> keys) the Registers Sub-Menu and press <ENTER>.

2)	Highlight the desired register set and press <ENTER>.  The register set choices are:

General Control & Status Registers

Sequencer Registers

CRT Controller Registers

Graphics Controller Registers

Attribute Controller Registers

Extended Registers

(Refer to the 65540/545 Data Sheet for a description of all registers)

3)	Highlight the desired register set to read its current value.

4)	To change the highlighted register's value, press <ENTER>, type in the new 2-digit hex value and press <ENTER>

5)	Press <ESC> to escape and return to the next highest menu

To read any of the 65540/545's video BIOS data values:

1)	Highlight (by moving the inverse video box with the <UP ARROW> or <DOWN ARROW> keys) the Bios Data Sub-Menu and press <ENTER>.

2)	Highlight the desired Bios Data address to read its current value.

3)	Press <ESC> to escape and return to the next highest menu

To Enable/Disable or configure the various features of the 65540/545:

1)	Highlight (by moving the inverse video box with the <UP ARROW> or <DOWN ARROW> keys) the Features Sub-Menu and press <ENTER>.

2)	Highlight the desired feature and press <ENTER> to configure each of the features:

Feature		Choices (Pressing <ENTER>)

Switch Display	Simultaneous/CRT Only/LCD Only

Use FP Mem & Dot Clk	Default/Programmed

Press D to Set Dot Clk	12 - 80 MHz

Press Z to Set Memory Clk	12 - 80 MHz

Smartmap	Off/On

Text Enhancement	Off/On

Inverse Video	Off/On

Color Reduction	NTSC/Equal Weighting/Green Only/Color

FRC Polynomial - MN	Increment the MN Value (2)

ACDCLK	Increment the ACDCLK Value (XR5E) (3)

FRC Level	2/4/8/16-level  or  NO/ 2 -level (TFT only)

Dithering	Off/256 Color Modes/All Modes

Turn On/Off Video	Off/On

8x19/8x30/8x32 Font	Enabled/Disabled

Optimal Compensation	Enabled/Disabled

Vertical Compensation	Enabled/Disabled

Vertical Centering	Automatic/Non-Automatic

Vertical Text Stretch	Disabled / DS+LI,DS,LI (4) (5)

Line Replication	Disabled / DS+LR,DS,LR / DS+LR,LR,DS (5)

Horiz. Compensation	Enabled/Disabled

Horizontal Centering	Disabled/Automatic (6)

Horizontal Doubling	Disabled/Enabled (6)

Sel. 1024x768x16-CRT	Interlaced/Non-Interlaced

72 Hz Refresh	Disabled/Enabled

Alt HSync Start - XR19	Increment Register XR19 (7)

Alt HSync End - XR1A	Increment Register XR1A (7)

Alt HTotal - XR1B	Increment Register XR1B (7)

Return to Main Menu (8)

Notes:

1)	The <ENTER> or <+> keys increment the clock to the next highest value, and

the <-> key decrements the clock to the next lowest value.

2)	The <ENTER> or <+> keys increment the register to the next highest value, and

the <-> key decrements the register to the next lowest value.  The <m> or <M> key also increments the "M" value, and the <n> or <N> key also increments the "N" value.  The <R> or <r> key resets the "MN" value to the original value.

3)	The <ENTER> or <+> keys increment the register to the next highest value, and the <-> key decrements the register to the next lowest value.  The <m> or <M> key also increments the least significant nibble value, and the <n> or <N> key also increments the most significant nibble value.  The <R> or <r> key resets the register to the original value.

4)	8x19/8x30/8x32 font must be enabled or else TF (Tall Fonts) support in the BIOS will always be disabled.  To load Tall Fonts externally, the 8x19/8x30/8x32 font should be disabled and (DS+TF,DS,TF) or (DS+TF,TF,DS) should be set.

5)	Vertical Compensation must be enabled or else LI (line insertion), TF (Tall Fonts) and LR (line replication) will be disabled.  The <+> or <-> key increments or decrements, respectively, the LI/LR registers.  The <R> or <r> key resets the LI/LR registers.

6)	Horiz. Compensation must be enabled or else these features are always disabled.

7)	The <ENTER> or <+> keys increment the register by a value of 2, and 	the <-> key decrements the register by a value of 2.  The <R> or <r> key resets the register to the original value.

8)	Pressing <ESC> also returns to the next highest menu.

9)	To read or write any of the 65540/545's Extended Register values from the Features sub-menu:

a)	Highlight (by moving the inverse video box with the <UP ARROW> or <DOWN ARROW> keys) the Features Sub-Menu and press <ENTER>.

b)	Type <X> and <R> keys followed by the 2-digit hex address and <ENTER>.

The current value of the selected Extended Register is displayed.

c)	Type in the new 2-digit hex value and press <ENTER>.

10)	To Program Memory or Dot Clock of the 65540/545 clock synthesizer Press Z or Press D 	respectively.

Type <D>  Or  <Z> key followed by the 2-digit  value  <ENTER>.

54XSET Program (sets the various 65540/545 graphic controller features from command line).

Usage: 54XSET [Feature] [Feature] ....  where [Feature] can be one or more of the following options:

Function	[Feature] Options

Switch Display	CT 	(CRT Only)

FP	(Flat Panel Only)

SM 	(Simultaneous Display)

CTZ 	(CRT Zoom Mode)

Inverse Video (Text)	IT	(Inverse)

TN	(Normal)

Inverse Video (Graph)	IG	(Inverse)

NG	(Normal)

Color Reduction	NT	(NTSC Weighting)

EQ	(Equal Weighting)

GR	(Green Gun)

Dithering	DP	(On In Pack Pxiel Modes Only)

DA 	(On In All Modes)

DO	(Off)

SMARTMAP	ES	(Enable)

DS	(Disable)

Text Enhancement	ET	(Enable)

DT	(Disable)

Optimal Compensation	OE	(Enable)

OD	(Disable)

Automatic Vertical Centering	EC	(Enable)

DC	(Disable)

Vertical Lines Replication	EL	(Enable)

DL	(Disable)

Tall Font	EF	(Enable)

DF	(Disable)

Automatic Horizontal Centering	HC	(Enable)

DH	(Disable)

Horizontal Doubling 	ED	(Enable)

DD 	(Disable)

Program Extended Register	XR??=	?? where first "??" indicates index address and

second "??" indicates data value (hex)

MODETEST Utility Program

This utility is a diagnostic used to set and display information for each and every video mode.  To execute the MODETEST program, type the following command at the command prompt:

Usage:  MODETEST [-m xx]

where [-m xx] is an optional field and [xx] defines the desired VGA mode.  (All the VGA modes will be sequentially displayed by pressing <ENTER> if the [-m xx] field is not used).

Press any key (except <ESC>) to display the next screen.  This will cycle through each video mode and display the following information:

Mode number

Resolution (in characters if text mode, or in pixels if graphics mode)

Number of available colors

Vertical scanning frequency

Horizontal scanning frequency

Dot Clock (pixel) frequency

It will also display a set of color bars to show off the colors and put a border of changing colors around the screen.

Press <ESC> while viewing any screen to exit and return to DOS

SETCLK Utility Program

This utility re-programs the 65540/545's MCLK (memory clock) and PCLK (dot clock) frequencies generated by the 65540/545 clock synthesizer (e.g.,  12 - 80 MHz).  To execute the SETCLK program, type the following command at the command prompt:

Usage:  SETCLK x yy[.zzz]

where:

x = m to reprogram the 65540/545's MCLK (memory clock)

= d to re-program the 65540/545's PCLK (dot clock)

yy = an integer value in MHz (decimal - not hex - value)

zzz = an fraction value in MHz (decimal - not hex - value)

Examples:  SETCLK  d 25.175

SETCLK  m 50.350

SETCLK  d 25

Caution:	Programming the MCLK and PCLK with incorrect values may cause unpredictable results and damage the display.

SETVGA Utility Program:  tc "The SETVGA utility program"\l§

This utility program is used to place your VGA adapter in emulation mode.  Your VGA adapter can emulate (imitate) the operation of other graphics adapters.  The following graphics standards can be emulated by your VGA adapter:

IBM Monochrome Adapter

IBM Color Graphics Adapter

IBM Enhanced Graphics Adapter

Hercules Monochrome Graphics Adapter

The SETVGA program is used to lock your VGA adapter into a specific emulation mode.  You would use the SETVGA program if you have software that is specifically written for these standards and cannot run in regular VGA mode.

Note:	Very few programs require the use of SETVGA.  Only when you execute certain old software packages do you need to use the SETVGA program.

To use the SETVGA utility to lock your VGA adapter into an emulation mode, you indicate the emulation mode to the SETVGA program by placing it after the name SETVGA on the command line.  The format for the command is:

SETVGA EMULATIONMODE <ENTER>

Valid values for EMULATIONMODE are any of the following:

Emulation mode	Description

MDA	Enables and locks MDA emulation

CGA	Enables and locks CGA emulation

EGAC	Enables and locks EGA color emulation

EGAM	Enables and locks EGA monochrome emulation

HERC	Enables and locks Hercules emulation

VGA	Disables emulations and returns to VGA operation

DEBUGVGA Utility Program

To execute DEBUGVGA, type the following command at the command prompt:

Usage:	DEBUGVGA

The DEBUGVGA prompt ($) will be displayed.  Type one of the following commands:

# 	Command	Description

00	PORT	Reads/Writes a byte from/to a port (Hex)

Usage:	PORT port_addr [value](hex)

01	PORTW	Reads/Writes a word from/to a port (Hex),

Usage:	PORTW port_addr [value](hex)

02	BYTE	Reads/Writes 8 bit values from/to a given address repeatedly

Usage:	BYTE seg:off length [value0] [value1]...(hex)

03	WORD	Reads/Writes 16 bit values from/to a given address repeatedly

Usage:	WORD seg:off length [value0] [value1]...(hex)

04	MOVE	Moves data from source to destination. Extended memory is supported

Usage:	MOVE sseg:soff length dseg:doff

05	VGAMODE	Sets VGA or SuperVGA BIOS mode

Usage:	VGAMODE vga_mode_no (hex)

06	MODETIME	Computes CRT timings for the current VGA mode (VGA only)

Usage:	MODETIME vga_mode_no (hex)

07	SR	Reads/Writes/Dumps VGA Sequencer registers at 3C4/3C5H

Usage:    SR[StartInd[EndInd]] [> file]

where	[StartInd]/[EndInd] = Starting/ending register number (0 - 4)

[> file] = Output file name

08	CR	Reads/Writes/Dumps VGA CRT Controller registers at 3D(B)4/3D(B)5H

Usage:    CR[StartInd[EndInd]] [> file]

where	[StartInd]/[EndInd] = Starting/ending register number (0 - 18)

[> file] = Output file name

09	AR	Reads/Writes/Dumps VGA Attribute Controller registers at 3C0/3C1H (VGA Type)

Usage:    AR[StartInd[EndInd]] [> file]

where	[StartInd]/[EndInd] = Starting/ending register number (0 - 14)

[> file] = Output file name

# 	Command	Description

10	GR	Reads/Writes/Dumps VGA Graphics Controller registers at 3CE/3CFH

Usage:    GR[StartInd[EndInd]] [> file]

where	[StartInd]/[EndInd] = Starting/ending register number (0 - 9)

[> file] = Output file name

11	VP	Reads/Writes/Dumps VGA External Palette at 3C7/3C8/3C9H

Usage:    VP[StartInd[EndInd]] [> file]

where	[StartInd]/[EndInd] = Starting/ending palette address (0 - FF)

[> file] = Output file name

12	XR	Reads/Writes/Dumps Chips Super VGA Extended registers at D6/3D7H

Usage:    XR[StartInd[EndInd]] [> file]

where	[StartInd]/[EndInd] = Starting/ending register number (0 - 7F)

[> file] = Output file name

13	MSR	Reads/Writes VGA Misceleneous output register at 3CC/3C2H

Usage:	MSR

14	FCR	Reads/Writes VGA Feature Control register at 3CA/3D(B)AH

Usage:	FCR

15	DUMPREGS	Dumps VGA and all Extended registers to Standard Output

Usage:	DUMPREGS [> file]

16	FILLPAT	Fills predefined pattern (0-7) already defined

Usage:		FILLPAT seg:off length pat_index (hex)

17	DEFPAT	Defines pattern which can be used with FILLPAT

Usage:	DEFPAT pat_name, width, height, data

18	VLINE	Draws a line in VGA memory using BIOS set pixel (Graphics Modes only)

Usage:	VLINE [X1 Y1] X2 Y2 [color]; (all numbers are in hex)

19	VRECT	Draws a rectagle in VGA memory using BIOS set pixel (Graphics Modes only)

Usage: 	VRECT [X Y] Width Height [color]; (all numbers are in hex)

# 	Command	Description

20	?	<command_name>	Display help messages

Usage:	?

The following screen is displayed:

00.	PORT	20. ?

01. PORTW	21. PAUSE

02. BYTE	22. LOAD

03. WORD	23. PROMPT

04. MOVE	24. ECHO

05. VGAMODE	25. SYS

06. MODETIME

07. SR

08. CR

09. AR

10. GR

11. VP

12. XR

13. MSR

14. FCR

15. DUMPREGS

16. FILLPAT

17. DEFPAT

18. VLINE

19. VRECT

Type Command No and then <CR> to get command usage. Esc to get out...

21	PAUSE   	Waits till key is pressed

Usage:	PAUSE

22	LOAD	command_file,			Runs commands from a file

Usage:	LOAD [file]

23	PROMPT	prompt_string,			Changes prompt string

Usage:	PROMPT

24	ECHO	Text to be displayed,		Displays text to VGA

Usage:	ECHO

25	SYS	word vga.txt, If SYS presedes any text string it is passed to DOS as it is

Usage:	SYS [text string]

54XDEMO Menu-Driven, User-Interactive Demo Program

The 54XDEMO program shows the various 65540/545 features, such as:

Simultaneous CRT Display With LCD Panels

SuperVGA Support on Non-Interlaced CRT Monitors:

-	640x480 256 Colors

-	800x600 256 Colors

-	1024x768 256 Colors

-	1024x768 16 Colors

High Resolution Flat Panel Support

Programmable Gray Scale Generation

Three color to grey scale reduction algorithms

64 grey scales on monochrome panels

Support For Color Flat Panels

Vertical Compensation

SMARTMAP

Text Enhancement

Inverse Video

Programmable clock operation

To execute the 54XDEMO program, type the following command at the command prompt:

Usage:	54XDEMO

The following screen should be displayed:

F65540/545 FLAT PANEL/CRT CONTROLLER

FEATURES DEMONSTRATION UTILITY

Copyright (c) 1993  Chips and Technologies, Inc.

Press <Enter> to continue...

Press <ENTER> and the following screen (the Main Menu) should be displayed:

CHIPS F65540/545 FLAT PANEL/CRT CONTROLLER

FEATURES DEMONSTRATION UTILITY

<F1>	SIMULTANEOUS DISPLAY of CRT and flat panel

<F2>	SUPER VGA SUPPORT on non-interlaced CRT monitors

<F3>	GRAY SCALING on monochrome panels

<F4>	THREE COLOR TO GRAY SCALE REDUCTION TECHNIQUES

<F5>	SMARTMAP

<F6>	VERTICAL COMPENSATION

<F7>	TEXT ENHANCEMENT and INVERSE VIDEO

<F8>	HIGH RESOLUTION FLAT PANEL SUPPORT

<F9>	COLOR GENERATION on color panels

ESC	EXIT (Return to DOS)

From this Main Menu:

Press <F1> through <F9> function keys to run the corresponding demo's Sub-Menu

(Hold down the ALT and T keys to run the Programmable Clocks demo Sub-Menu)

Press ESC key to exit the demo and return to DOS

From each demo's Sub-Menu:

Press ESC key to exit the demo and return to the Main Menu

Press one of the letters and numbers 	indicated on the Sub-Menu to perform the corresponding function

-	<P> "Problem & Solution" provides a brief explanation of each feature

-	Some Sub-Menus provide <ENTER> as an option to display the next screen

(which while viewing you can activate the functions listed on the Sub-Menu)

Example:

From the Main Menu, press <F3> to access the GRAY SCALING demo's Sub-Menu

Press <P> to read about how programming the 65540/545's polynomial FRC  register (XR6E) enables the 65540/545 to reduce flicker on fast-response LCDs

Press <ESC> to return to the GRAY SCALING demo's Sub-Menu

Press <ENTER> to access the next image (which is 640x480 16 colors).

While viewing the image:

a)	Press M to decrement the Polynomial M value in XR6E

b)	Press N to decrement the Polynomial M value in XR6E

c)	Press D to turn dither on/off

d)	Press F to change the frame rate control from 2- to 4- to 8- to 16- levels or No to 2

in case of  TFT  Color panel

Press <ENTER> to access the next image (which is 640x480 256 colors).

While viewing the image, repeat a) through d) above.

Press <ENTER> to access the next image (which is 640x480 16 colors).

While viewing the image, repeat a) through d) above.

Press <ENTER> to access the next image (which is 320x200 256 colors).

While viewing the image, repeat a) through d) above.

Press <ENTER> (or <ESC>) to return to the GRAY SCALING demo's Sub-Menu

Press <ESC> to return to the Main Menu

Technical information

The 65540/545 demo and utilities require the Chips and Technologies' 65540/545 or 65540/545 VGA Flat Panel/CRT Controller .